BWSN design objectives calculation example – "Case A"

The Excel file shows the results of the four objectives for Network #1 and for a single injection event.

The injection event:

- Injection junction: JUNCTION-88

- Injection time: 60th quality time step – since the quality time step is 5 minutes the injection event time is 5:00

- Injection duration: 2 hours

- Injection mass flow: 479167 mg/minute – (125 l/hr X 230000 mg/L) / 60 = 479167

The result data for Z1, Z2, Z3 and Z4 is shown in the rows of the Excel file (for simplicity reasons one should first tests Z4):

Z4 – a value of 1 indicates that the pollution has reached the specified node. One should see that over all the pollution reaches only 10 junctions in the following order: 88, 87, 79, 80, 82, 85, 83, 81, 84 and 86.

Z1 – the values presented for each node is the time, in minutes prior to the pollution detection in the node, that the pollution has reached the node after the injection time. For example: for junction 88 the pollution is noticed one time step after the injection, 5 minutes. For nodes that the pollution did not reach a value of twice the maximum simulation duration is used (2 X 96 hours X 60 min/hr = 11,520). This is not a penalty – it is simply a flag!

Z2 – the population effected by the contamination prior to the pollution detection in the node. One should note that for junctions 88, 87, 79 and 80 the value is 0. This is due to the fact that there is no demand associated with these junctions. The first people affected are at junction 82. For the computation of Z2, the daily average demand, at each node, is required. Since the total simulation time is 96 hours and each 24 hours have different demand, the average is calculated as the total demand volume over 96 hours divided by 4. A value of -1 is used for nodes that the contamination did not reach. This is not a penalty – it is simply a flag!

Z3 – the expected volume, in gallons, of consumed contaminated water prior to the pollution detection in the node. One should note that for junctions 88, 87, 79 and 80 the value is 0. This is due to the fact that there is no demand associated with these junctions so no contaminated water is consumed. A value of -1 is used for nodes that the contamination did not reach. This is not a penalty – it is simply a flag!